🌐 Cisco Packet Tracer — Two-Network Router Configuration

📌 Project Overview

This project demonstrates how a Cisco router connects two separate IPv4 networks using Cisco Packet Tracer.

The lab was built from scratch to practice fundamental networking concepts including:

- IPv4 addressing

- Subnetting

- Default gateways

- Router interface configuration

- Connected routes

- Network troubleshooting

- End-to-end connectivity testing

🗺️ Network Topology

CISCO ROUTER

┌────────────────────┐

│                    │

Fa0/0  │                    │  Fa1/0

│                    │

└───────┬─────┬──────┘

│     │

SW1   SW2

│     │

PC1   PC2

Network Layout

192.168.10.0/24                         192.168.20.0/24

PC1                                      PC2

│                                        │

│ 192.168.10.10                          │ 192.168.20.10

│                                        │

SW1                                      SW2

│                                        │

│                                        │

└──── Fa0/0 ── ROUTER ── Fa1/0 ────────┘

│          │

192.168.10.1  192.168.20.1

📊 IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | FastEthernet0 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Router | FastEthernet0/0 | 192.168.10.1 | 255.255.255.0 | — |
| Router | FastEthernet1/0 | 192.168.20.1 | 255.255.255.0 | — |
| PC2 | FastEthernet0 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |

🔧 Technologies & Concepts

⚙️ Router Configuration

The router was configured with two FastEthernet interfaces.

Interface 1

interface FastEthernet0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

Interface 2

interface FastEthernet1/0

ip address 192.168.20.1 255.255.255.0

no shutdown

🔍 Verification & Testing

1. Interface Verification

show ip interface brief

Used to verify that the router interfaces were correctly configured and operational.

Expected result:

FastEthernet0/0    192.168.10.1    up    up

FastEthernet1/0    192.168.20.1    up    up

2. Routing Table

show ip route

The router automatically learned both networks as directly connected routes.

C    192.168.10.0/24

C    192.168.20.0/24

3. End-to-End Connectivity

From PC1:

ping 192.168.20.10

A successful response confirmed connectivity between the two different networks.

🧠 What I Learned

Through this lab, I learned how to:

- Configure Cisco router interfaces

- Assign IPv4 addresses

- Understand /24 subnetting

- Configure default gateways

- Enable interfaces using no shutdown

- Read show ip interface brief

- Read a Cisco routing table

- Understand directly connected routes

- Troubleshoot physical connectivity

- Verify network connectivity using ping

- Understand how routers forward traffic between different networks

🛠️ Troubleshooting

During the lab, an interface initially showed a down state.

The issue was traced to the physical cable connection between the router and switch.

After correcting the connection and enabling the interface with:

no shutdown

the interface became operational.

This demonstrated an important real-world troubleshooting principle:

Always check the physical layer before assuming the configuration is wrong.

📁 Project Structure

cisco-day-03-router-routing/

│

├── 📄 README.md

├── 📄 day3-router-routing.pkt

├── 📄 router-config.txt

├── 🖼️ topology.png

│

└── 📁 screenshots/

├── interfaces.png

├── routing-table.png

└── successful-ping.png

🎯 Project Objective

The primary objective was to build and configure a small routed network from scratch and demonstrate successful communication between two different IPv4 networks.

Result

✅ Successfully completed

PC1:

192.168.10.10

was able to communicate with PC2:

192.168.20.10

through the Cisco router.

📚 Skills Demonstrated

🚀 Next Steps

This project is part of my 30-Day Junior Network Engineer Portfolio.

Upcoming labs will progressively cover:

- DHCP

- VLANs

- Trunking

- Inter-VLAN Routing

- Static Routing

- OSPF

- Network Troubleshooting

- Enterprise Network Design

👨‍💻 Author

Junior Network Engineering Portfolio

Building practical networking skills through hands-on labs and progressively complex Cisco Packet Tracer projects.